B.Sc. DEGREE EXAMINATION, NOVEMBER 2015.

II YEAR — IV SEMESTER

Major Paper VIII — ELECTRICITY AND

MAGNETISM

Time : 3 hours	    Max. Marks : 60

SECTION A — (10 × 1 = 10 marks)

Answer any TEN questions

- Define electric flux. Give its unit.

- What is meant by spherically symmetric charge distribution?

- Why is copper wire not suitable for a potentiometer?

- A 10 Ω resistance is connected in series with a cell of emf 10V. A voltmeter is connected in parallel to a cell and it reads 9.9V. Find internal resistance of the cell.

- Define time constant of an L-R circuit.

- What is resistance?

- What is seebeck effect?

- Define peltier effect.

- Define magnetic susceptibility.

- Define magnetization.

- What do you mean by pyro-electricity?

- Mention the classification of magnetic materials.

SECTION B — (5 × 4 = 20 marks)

Answer any FIVE questions

- State and explain Gauss’s law.

- Explain the principle of potentiometer.

- Describe with full theory the method of measuring a high resistance by the leakage method.

- Describe the laws of thermo emf.

- Write the properties of ferromagnetic materials.

- Write short notes on Antiferromagnetism and Ferrimagnetisms.

- Describe the application of Gauss law to an insulated conductor.

SECTION C — (3 × 10 = 30 marks)

Answer any THREE questions

- Apply Gauss’s law to calculate the electric field due to a uniformly charged sphere at points:

  - Outside the sphere

  - At the surface of the sphere

  - Inside the sphere

- What is potentiometer? How will you use it to calibrate an ammeter and a voltmeter?

- Describe the growth and decay of charge in a C – R circuit.

- Describe how you would determine the peltier co-efficient of a junction experimentally.

- Derive the relation between three magnetic vectors B, H and M.

———————