B.Sc. DEGREE EXAMINATION, APRIL 2017.

II YEAR — IV SEMESTER

Major Paper IV — MICROPROCESSORS AND ITS APPLICATIONS

Time : 3 hours	    Max. Marks : 60

SECTION A — (10 × 1 = 10 marks)

Answer any TEN questions.

- What are the functions of an accumulator?

- List out the two categories of the 8085 instructions.

- What is the use of addressing modes? Mention the different types.

- Give the difference between JZ and JNZ.

- What is CALL instruction?

- What is time delay?

- Define DAA instruction.

- Write an assembly language program to add two BCD numbers.

- What is a TRAP?

- What are the two signals available on the 8085 for DMA controller?

- What is Accumulator?

- Define DMA.

SECTION B — (5 × 4 = 20 marks)

Answer any FIVE questions.

- Explain the function of ALU and IO/M signals in the 8085 architecture?

- List the various addressing modes present in 8085.

- Write a program to count continuously in hexadecimal from FFH to OOH in a system with a 0.5 micro second clock period. Use registers C to set up a one millisecond delay between each count and display the numbers at one of the output ports.

- Write an assembly language program to convert BCD data to binary using 8085.

- List the priority interrupts of 8085.

- If the 8085 adds 87H and 79H, specify the contents of the accumulator and the status of the S, Z, and CY flag.

- Give the functional categories of 8085 instructions.

SECTION C — (3 × 10 = 30 marks)

Answer any THREE questions.

- Explain in detail 8085 instruction set with an example.

- Discuss in detail dynamic debugging.

- Explain in detail branching instruction with an example.

- To write an assembly language program to convert binary code in to ASCII code.

- How to interface a DMA controller with a microprocessor? How DMA controller transfers large amount of data from one memory locations to another memory location? Explain .

—————